| Document properties |  |
| --- | --- |
| Date, Version | 17 March 2022 V0.2 |
| Department, Area | Operational Excellence, Glencore Copper |
| Prepared by | Tony Archer |
| Reviewed by | Rohan Reisener |
| Comment/s | Draft |

Preface

The Asset Management Framework, Policy, Manuals, Standards and guidelines are intended to provide leading Asset Management practices when applied and will be continually improved for the benefit of Glencore Copper.

The real value of sharing the knowledge gained from the use of these documents will come from continued discussion of the issues and rationale behind each document rather than by reading and interpreting them as individuals.

Readers are encouraged to consider these against our values around Safety, Responsibility, Simplicity, Integrity,

Openness and Entrepreneurialism and how they reflect our Purpose – to responsibly source the commodities that

advance everyday life. This document lays out the minimum standard for Glencore Copper and compliance to this procedure will demonstrate a maturity value equal to Planned/Deployed (3) on the Glencore Copper Asset Management Maturity Scale.

# Table of contents

Preface	2

Table of contents	2

Figures	2

Tables	2

1. Introduction	3

2. Objectives	3

3. Definitions	4

4. Approach	5

5. Deployment	13

6. Improvement	14

7. Spares Criticality Review	14

8. References	15

9. Revision History	15

Figures

Figure 4-1 Spares Criticality Analysis Process Flow	6

# Tables

Table 4-1 Spares Criticality Analysis Process Description	7

Table 4-2 FUNCTIONAL DESCRIPTIONS	12

Table 8-1 References	15

# Introduction

The Asset Management Framework defines how Glencore Copper manages the whole of life of an asset to ensure optimal value is extracted and to meet business and stakeholder objectives.

The Framework is supported by a hierarchy of documentation starting with a Manual for each Framework element along with Standards or Guidelines to ensure the requirements of the Framework are consistently met across Glencore Copper.

This Guideline provides the process for determining spares criticality and complements the Asset Criticality Analysis Standard, Element 05 Equipment Operating and Maintenance Strategy Manual and Element E13 Procurement and Supply Chain Management Manual.

# Objectives

The objective of the Spares Criticality Analysis Guideline is to define the process to be followed to determine which SAP materials are to be considered as Critical Spares and the most appropriate way of dealing with those spares.

## Accountabilities

Dedicated resources must be assigned, and capabilities developed within each Operation to identify the key aspects required to ensure all spares required for operating and maintaining Glencore Copper assets are reviewed to determine if the material or spare is critical and how the spare is to be managed.

Accountability must be assigned for each and all of the following:

- Determining materials where used.

- Impact of component/spare failure on equipment performance.

- Cataloguing of material.

- Determining material lead time.

- Supplier/vendor stock negotiations.

- Managing catalogued materials.

## Scope

When assessing risk, Element E14 Risk Management and Compliance manual should be consulted together with the Glencore Copper Corporate Risk Management Framework, which provides a methodology for defining the risks related to business uncertainty. It utilises a well-defined consequence matrix to determine the impacts of uncertainty regarding safety, environment, production, failure to meet regulatory and legal requirements and reputation.

It also utilises a well-defined consequence rating combined with a simple likelihood factor that when combined with the consequence factor provides a risk ranking, which in turn provides quantifiable means by which asset system and asset risk mitigation can be prioritised.

The criticality of assets translates to the maintenance and operational activities associated with the assets as well as the spares and materials used in the maintenance and operation of the assets.  While there may be a number of spares or materials used in the ongoing maintenance and operation of assets, the unavailability of certain spares when needed for scheduled maintenance or due to an unplanned event, could lead to the failure of mitigation activities developed for critical assets and leave the organisation exposed to significant risk.

The scope of Critical Spares Analysis include:

- Determining those spares or materials that are critical to ensuring the availability and maintainability of critical assets.

- Determining the most appropriate way to manage these spares.

The ongoing management of spares is achieved through established Supply processes which include Inventory, Procurement, Warehousing etc.  The scope of this guideline does not extend into these processes.

# Definitions

Asset – an item, thing or entity that has potential or actual value to an organisation.

Catalogued Material – a material, including spares, that has been set up in SAP with the required Material Master ERSA and HERS records.

Criticality Analysis - is a process by which assets are assigned a criticality rating based on their potential risk.

Critical Asset – an asset having potential to significantly impact on the achievement of the organisation’s objectives”.  Assets that have a criticality ranking of A derived through the Asset Criticality Analysis Standard.

Critical Spare – a part used in a critical asset, which if it fails and cannot be replaced within reasonable time, will lead to an unacceptable level of risk for Glencore Copper. Categorised as C1 (Major Disruption), C2 (Significant Disruption) and C3 (Non-Critical).

Functional Failure – is the inability of a system to meet a specified performance standard, and also includes the inability to function at the level of performance that has been specified as satisfactory.

High-Risk Event – events that cause a risk impact of >17 on the Glencore Copper Asset Criticality Risk Matrix.

Preservation Maintenance – periodic maintenance done on spares while in storage to ensure the integrity of the spare is maintained and it can reliably perform its function at any time when required.

Spares Criticality Analysis – a process by which spares, or materials are evaluated to determine if they are critical or not and the most appropriate way to manage them.

Service Level - the percentage likelihood that the users will find the materials they need, in the quantities they need, available in stock, when they look for them.

# Approach

The approach for Spares Criticality Analysis is described through:

- Process Flow - pictorially illustrates the key Spares Criticality Analysis activities, refer to Figure 4-1.

- Process Description – defines in more detail the key Spares Criticality Analysis activities, refer to Section 4.2.

- Process RACI Matrix - indicates for each activity, the functions, who are Responsible, Accountable, who are Consulted prior to final decision/action, and those who are Informed of any outcomes and actions, refer to Section 4.3.

## Process Flow

Figure 4-1 Spares Criticality Analysis Process Flow

## Process Description

Table 4-1 Spares Criticality Analysis Process Description

| PROCESS STEP | DESCRIPTION |
| --- | --- |
| 1.1 Spare used on Critical Assets | Determine whether the spare being assessed is used on or associated with an asset has been identified as a Critical Asset (A). Some materials are used on more than one asset, and it is important to ensure all instances are reviewed as similar assets may have different Criticality ratings. Depending on the completeness of Bill of Materials (BOMs) for assets in SAP, a Materials Where Used List (IW13) could provide information, otherwise the BOM should be sourced from the supplier. Although a review of past use of materials on workorders may provide input, this should not be heavily relied upon as some critical or insurance spares will have limited or no movement. If the spare is not used on any Critical Asset (A), then it will not be considered a Critical Spare – go to step 1.2. If it is used on a Critical Asset (A) then go to step 1.4. |
| 1.2 Spare already catalogued (C) | Determine if the spare is already a catalogued material. If it is already a catalogued material in SAP, confirm it is tagged with a Non-Critical (C) rating, minor disruption, on the material master. If the spare is already catalogued correctly, then go to step 1.10. If the spare has not been catalogued or catalogued incorrectly, then go to step 1.3. |
| 1.3 Catalogue Spare (C) | The Reliability team are responsible for sourcing the required information and completing the necessary request documentation to instigate the cataloguing process to be completed by the Cataloguer. Refer to Glencore Copper “Catalogue New Stocked or Non-Stocked Material Reference Guide”. Catalogue a new material or update the details of an existing material to reflect a Non-Critical (C) in the material master. The information that must be provided to support the requirements of the Glencore Inventory Management Policy include: Supply Planning: - Inventory grouping - SAP Standard grouping - Materials Requirements Planning (MRP) Type - Management group - Lifecycle - Movement Class - Criticality. For non-critical spares, assign the Criticality as C and the Management Group to be either Spare or Consumable. The other data elements will depend on various considerations. These should be determined based on the requirements of the Inventory Management Policy and with input from Supply Chain. Ordering Parameters include: - Lead Time - Replenishment Cycle - Service Level. The Service Level is dependent on the Forecastability of the spare and ranges from 50% to 95%. The ordering parameters must be determined in consultation with Supply Chain. For new materials, the expected demand must also be determined to calculate: - Lead Time Stock - Safety Stock - Cycle Stock - Re-order and Order-up-to Points. The methods for calculating these values are detailed in the Inventory Management Policy. Go to step 1.10. |
| 1.4 Spare failure causes functional failure of Critical Assets | The effect of a failure of the specific spare or material being assessed is to be considered. Not every spare used on an asset will result in a high-risk event if it fails. Only those spares that will trigger a high-risk event when they fail are to be considered as Critical. The Failure Modes identified through the Glencore Copper Maintenance Tactics Development process for the asset should provide the link between the performance of the asset and the spare being analysed. Should a spare failure result in system/asset functional failure then go to step 1.9. Spares whose failure do not trigger a high-risk event are to be further assessed against other contributing factors. Go to step 1.5. |
| 1.5 Extended spare unavailability increases risk ranking of asset failure | Some spares used on critical assets may not trigger the high-risk event that was used to determine the criticality of the asset. Though an extended period of unavailability of the spare could increase the consequence associated with a failure of the spare if it cannot be replaced in a timely manner. For example, in a redundant system configuration, it would be prudent to assess the duty equipment as non-critical, as the redundant system/equipment would provide functional capability in the event of duty failure. In this scenario we would need to ensure the duty function is restored within an acceptable period. Failure to do so would lead to an increase in risk exposure because of the system/equipment operating in a non-redundant configuration for an extended period. Another example, a failure of a component could lead to reduced overall system performance, which could be tolerated over a short period, but if the situation is prolonged, it will lead to unrecoverable losses. If a required spare is not available within the desired time frame, then the criteria that was used to assess the original risk rank could be considered to have changed and should be reviewed. If an extended unavailability of the spare does not increase the risk rank to the Criticality trigger, then the spare should be considered as non-Critical, then go to step 1.2, else step 1.6. |
| 1.6 Spare already catalogued (B) | Determine if the spare is already a catalogued material. If it is already a catalogued material in SAP, confirm it is tagged with a Significant Disruption (B) rating on the material master as a prolonged unavailability of the spare will cause recoverable losses or increased risk. If the spare is already catalogued correctly, then go to step 1.10. If the spare has not been catalogued or catalogued incorrectly, then go to step 1.7. |
| 1.7 Catalogue Spare (B) | Refer to process step 1.3 except for the Service level which should be set at 98%. Based on the Inventory Management Policy, the ordering parameters must be determined in consultation with Supply Chain. Go to step 1.10. |
| 1.8 Spare already catalogued (A) | Determine if the spare is already a catalogued material. If it is already a catalogued material in SAP, confirm it is tagged with a Major Disruption (A) rating on the material master, as unavailability of the spare will cause unrecoverable losses or unacceptable risk. If the spare is already catalogued correctly, then go to step 1.10. If the spare has not been catalogued or catalogued incorrectly, then go to step 1.9. |
| 1.9 Catalogue Spare (A) | Refer to process step 1.3, except for the Service level which should be set at 99%. Go to step 1.10. |
| 1.10 Vendor Managed Stock option available and feasible | The carrying cost of a spare includes the purchase price of the spare and the holding cost. Holding cost include such items as the financial impact on working capital, potential preservation maintenance and warehousing costs. site Finance should be able to provide guidance on these financial impacts. Generally, the cost of holding a spare would be in the order of 20% of the purchase price of the spare annually. As some critical spares tend to be of very high value and low moving, together with the fact the spare holding cost is more than just the purchase price, the feasibility of Vendor Managed Stock options should be considered. It is unlikely suppliers will provide this option for free, but it may be a more economical option for site rather than an outright purchase and hold strategy. Whilst supplier managed agreements could provide a secure strategy, other alternatives such as group (area or region) held spares should also be considered where feasible. The purchasing and expediting time for supplier held stock should be considered to ensure these do not result in a potential increase in risk. If Vendor Managed Stock is not a feasible option, then go to step 1.11, if it is then process step 1.13. |
| 1.11 Spare requires preservation maintenance? | All spares, especially those that meet the criteria to be classified as Critical Spares should be evaluated to determine whether the spare will require some form of preservation maintenance whilst they are in storage. Should such maintenance be required the correct tactics should be developed using the Glencore Copper Maintenance Tactics Development standard. If the spare will require maintenance while in storage, then go to step 1.13, else step 1.15. |
| 1.12 Develop Preservation Maintenance Strategy (E05 Maintenance Tactics Development) | Develop the required maintenance strategy for the spare part as per the Spare Preservation Strategy Guideline. It is important to note that the failure modes for stationary components in storage will be very different than for those in operation. Go to step 1.14. |
| 1.13 Establish and manage Vendor agreement (E13 Procurement and Supply Chain Management Manual) | The arrangement with the vendor must be formalised. This is especially important for materials that have been identified as Critical. This is done to ensure all parties involved understand what is expected and what they are accountable for. This should cover at least the following: - The communication protocol in the event of changes in planned stockholding or demand. - Changes in lead times. - Compensation related to the arrangement. E13 Procurement and Supply Chain Manual Section 4 Process Step 1.3 with regards to vendor holding agreements stipulates: As a minimum the spares strategy should detail the spare parts holding agreements, where Glencore Copper receive a benefit from such agreements, like: - On consignment – where Glencore Copper hold the stock but do not pay until they are required. The supplier owns the stock until Glencore purchases. - Supplier managed – where suppliers manage the ordering of stock on Glencore Copper’s sites behalf and either site hold the stock, or the supplier does. - Standing orders – which stipulates a standard purchase price and quantity. - Forward purchasing agreements – used to lock in a specific price in the agreement period to reduce the risk of price volatility. Go to step 1.14. |
| 1.14 Manage Spare (Copper Global Inventory Planning Policy v3.2) | Spares are managed using the standard Glencore Copper supply managed processes such as Inventory, Procurement, Warehousing, etc. based on the way the materials have been set up in SAP. Refer to Glencore Copper Global Inventory Planning Policy (v3.2). |

## Process RACI Description

The RACI matrix below in Table 4-2 addresses the high-level functional elements of the Spares Criticality Analysis process.

Site roles may differ to the roles identified in this RACI, these roles however should be used as a guide for the development of a site-specific RACI, while maintaining the intent and content of this guideline.

Note:	The HSE and Commercial Departments do not appear in this high-level RACI, although their involvement will be required from time to time dependent on the nature of the variance.

The codes used in the RACI analysis are defined as follows:

Table 4-2 Process RACI

| R | Responsible | The “doer” – the individual(s) who will perform an activity or directs others to perform the activity. |
| --- | --- | --- |
| A | Accountable | “The buck stops here” –the individual who ultimately “owns” the activity – there should only be one person. |
| C | Consulted | person that needs to feedback and contribute to the activity. |
| I | Inform | The individual(s) who needs to be informed after a decision or action is taken if relevant. (via 1-way communication). |

| Functional Role Process Step | Reliability Management | Maintenance Management | Maintenance Execution | Reliability Engineering | Maintenance Planning | Technical Services | Supply Management | Supply & Procurement |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.1 Spare used on Critical Assets | A | C | C | R | C | C |  |  |
| 1.2 Spare already catalogued (C3) | A |  |  | R | C |  |  | C |
| 1.3 Catalogue Spare (C3) |  | C | C | R | C | C | A | R |
| 1.4 Spare failure causes functional failure of Critical Assets | A | C | C | R | C | C | I | I |
| 1.5 Extended spare unavailability increases risk rank of equipment failure | A | C | C | R | C | C | I |  |
| 1.6 Spare already catalogued (C2) | A |  |  | R | C |  |  | C |
| 1.7 Catalogue Spare (C2) |  | C | C | R | C | C | A | R |
| 1.8 Spare already catalogued (C1) | A |  |  | R | C |  |  | C |
| 1.9 Catalogue Spare (C1) |  | C | C | R | C | C | A | R |
| 1.10 Vendor Managed Stock option available | I |  |  | C |  | C | A | R |
| 1.11 Spare requires preservation maintenance | A | C | C | R |  | C | I | I |
| 1.12 Develop Preservation Maintenance Strategy | A | C | C | R | C | C | I | I |
| 1.13 Establish and manage Vendor agreement | I | I |  | C | I | C | A | R |
| 1.14 Manage Spare |  |  |  |  |  |  | A | R |

Table 4-2 FUNCTIONAL DESCRIPTIONS

| Function | Description |
| --- | --- |
| Reliability Management | - Ensures all materials or spares are reviewed to ensure Critical Spares have been identified. - Ensures Spares Criticality is reviewed after any change in classifications from a Criticality Assessment of Criticality Review. - Ensures the Spares Criticality Analysis facts and assumptions are reviewed annually for all Critical Spares. - Provides input as when required to support other groups in the implementation of Spares Criticality Analysis. |
| Maintenance Management | - Provides input as and when required for the implementation of Spares Criticality Analysis. - Ensures preservation maintenance is done as and when required. |
| Maintenance Execution | - Provides input as and when required for the implementation of Spares Criticality Analysis. - Executes preservation maintenance on Critical Spares. |
| Reliability Engineering | - Responsible for implementation of the Spares Criticality Analysis process as and when required. - Provides input as when required to support other groups in the implementation of Spares Criticality Analysis. |
| Maintenance Planning | - Provides input as and when required for the implementation of Spares Criticality Analysis. |
| Technical Services – Metallurgists, Process Engineers and Mining | - Provides input as and when required for the implementation of Spares Criticality Analysis. |
| Supply Management | - Provides input as and when required for the implementation of the Spares Criticality Analysis. - Ensures Supply Function processes are in place and adequate to manage Critical Spares. |
| Supply Functions | - Manages Critical Spares using relevant and appropriate Supply Processes such as Purchasing, Inventory Management, Warehousing, etc. - Provides input as when required to support other groups in the implementation of Spares Criticality Analysis. |

# Deployment

The deployment of the Spares Criticality Analysis approach across all business areas is through:

- Awareness presentation

- Position description alignment

- Competency assessment

- Training, coaching, and mentoring.

## Awareness Presentation

This guideline is supported by a slide presentation and video to help facilitate communication and to raise awareness on how it contributes to the Equipment Operating and Maintenance Strategy and business success.

## Position Description Alignment

To effectively deploy this guideline, the essential process steps and activities need to be diligently mapped and assigned to approved positions within Glencore Copper.

The RACI matrix assists in the visualisation that everyone is working towards the same goal.  It clarifies, not only the individual’s responsibilities, but also that of others.  It has proven to be an effective tool to quickly understand and close the gaps between current and desired practices and business process activities.

Position descriptions including KPIs need to be aligned to the RACI matrix and measures of success. This is the basis for ongoing performance reviews and the creation and execution of individual development plans.

## Competency Assessment

This guideline is supported by an Asset Management competencies assessment process and each site is responsible for assessing Asset Management competencies:

  - Guide recruitment, selection, development, and deployment of people in asset management roles across the company

  - Provide employees responsible for managing assets with feedback, development, coaching and recognition for their performance.

  - Map, build and manage the asset management talent pool across the company.

  - Identify strengths and development needs in asset management practitioners.

## Training, Coaching and Mentoring

Coaching and mentoring are another way to assist employees improve their understanding of their role and to improve performance.

Coaching is generally about improving performance in the short term where mentoring is about achieving high performance and career aspirations over a longer term.

Coaching or mentoring requires addressing a person is not currently up to the required level to perform their role as effectively and efficiently as they could.  The build-up of skills to perform efficiently and effectively requires a large investment of time and cooperation.

Coaches and Mentors need to be carefully selected and must be committed for the long term to effect maximum success.

# Improvement

The process to review and continually improve the approach and deployment for Spares Criticality Analysis is scheduled annually or when there is a sudden change in the Business and Asset Management objectives. As the organization matures a longer review period could be considered.

# Spares Criticality Review

A Spares Criticality Analysis should be undertaken annually for Critical assets after any changes to Asset Criticality from the annual review of Critical A items or when significant changes occur in the business that could prompt a change in criticality ranking, for example the introduction of redundancy in Processes and Sub Processes.

The purpose of the annual review is to ensure any changes outside of Glencore Copper’s control that could affect spare availability, i.e., supply source or lead time, is accounted for.  The impact on Critical Spares classification of any changes should also be assessed as they happen to ensure new Critical Spares are identified and are available when needed, but also to identify spares that would no longer be needed as Critical Spares.

# References

The following documents provide detailed support material that should be read in conjunction with this guideline to provide a comprehensive understanding of Spares Criticality Analysis.

Table 8-1 References

| Document Title | Description |
| --- | --- |
| Asset Management Framework | This document defines the overall direction of asset management across Glencore Copper. |
| E05 Equipment Operating and Maintenance Strategy Manual | This document defines the process for the development and implementation of Equipment Operating and Maintenance Strategies within Glencore Copper. |
| Maintenance Tactics Development Standard | This document defines |
| Glencore Copper Criticality Analysis Risk Matrix | The multi-criteria risk matrix that is used to determine corporate risk through consequence and likelihood evaluation of potential failure events. |
| Asset Criticality Analysis Standard | The process that is used to determine Asset Criticality. Asset Criticality is a key decision-making tool used in the determination of Spares Criticality. |
| Copper Global Inventory Planning Policy (v3.2) | This document defines the inventory management policies and includes a description of the physical system setup as well as the technical parameters along with a clear explanation of the purpose of these parameters in terms of managing materials in inventory and the supply chain. |

# Revision History

| Version No. | Reason for Revision | Date |
| --- | --- | --- |
| 0.1 | New Guideline to support Asset Management Manual | 06/12/2021 |
| 0.2 | Gather Documents and Create Draft Guideline | 17/01/2022 |
| 0.3 | Review and Signoff 1st Draft Guideline with Sites | 17/03/2022 |
| 0.4 | Reformat and Issue for Final Review |  |
| 1.0 | Update Site Signoff and Publish |  |